SWITCH ROUTER LAB

Cisco Packet Tracer Network Configuration & Analysis

Source File: Switch Router.pkt

Two Layer-2 LANs connected through a single Layer-3 router

# 1. Introduction

This lab demonstrates basic LAN switching and inter-network routing using Cisco Packet Tracer. Two Cisco 2960-24TT switches provide Layer 2 connectivity for six PCs. A Cisco 2911 router, hostname DSU1, connects the two /8 networks and routes traffic between them.

# 2. Objectives

Connect multiple PCs to Layer 2 switches.

Configure IPv4 addressing on router interfaces and end devices.

Use default gateways to enable communication beyond each local subnet.

Understand how a router forwards traffic between directly connected networks.

Verify connectivity between hosts on separate LANs.

Distinguish Layer 2 switching from Layer 3 routing.

# 3. Router Substitution Note

The original Router1 in the Packet Tracer source was a generic Router-PT with two FastEthernet interfaces. It was re-exported as a Cisco 2911 while retaining the hostname DSU1, the two active interfaces, and their IP configuration. The active interfaces were renamed to GigabitEthernet0/0 and GigabitEthernet0/1 to match the 2911 hardware. Unused serial/fiber interfaces and enable password boilerplate were not carried over.

# 4. Network Topology

LAN1 uses the 10.0.0.0/8 network and LAN2 uses the 20.0.0.0/8 network. DSU1 is the Layer 3 boundary between the two LANs.

## Logical path

LAN1 PCs → Switch0 → DSU1 → Switch1 → LAN2 PCs

# 5. Devices and Roles

| Device | Model | Role |
| --- | --- | --- |
| Switch0 | 2960-24TT | Layer 2 access switch for 10.0.0.0/8 |
| Switch1 | 2960-24TT | Layer 2 access switch for 20.0.0.0/8 |
| Router1 / DSU1 | Cisco 2911 | Inter-LAN Layer 3 router |
| PC0–PC2 | PC-PT | Hosts on LAN1 |
| PC3–PC5 | PC-PT | Hosts on LAN2 |
| Power Distribution Device0 | iot_pdu | Present but unconnected |

# 6. IP Addressing Plan

| Subnet | Router Interface | Router IP | Hosts | Default Gateway |
| --- | --- | --- | --- | --- |
| LAN1: 10.0.0.0/8 | Gi0/0 | 10.0.0.1 | PC0 .2, PC1 .3, PC2 .4 | 10.0.0.1 |
| LAN2: 20.0.0.0/8 | Gi0/1 | 20.0.0.1 | PC3 .2, PC4 .3, PC5 .4 | 20.0.0.1 |

# 7. Routing Design

No static routes or dynamic routing protocols are required. DSU1 has an interface in each LAN, so both networks appear as directly connected routes in its routing table.

10.0.0.0/8 → directly connected via Gi0/0
20.0.0.0/8 → directly connected via Gi0/1

When a PC sends traffic to the other LAN, it forwards the packet to its default gateway. DSU1 receives the packet and forwards it through the interface belonging to the destination subnet.

# 8. Router Configuration — DSU1

hostname DSU1
interface GigabitEthernet0/0
 ip address 10.0.0.1 255.0.0.0
 duplex auto
 speed auto

interface GigabitEthernet0/1
 ip address 20.0.0.1 255.0.0.0
 duplex auto
 speed auto

interface GigabitEthernet0/2
 no ip address
 shutdown

# 9. Switch Configurations

## Switch0

hostname Switch0
interface FastEthernet0/1   ! PC0
interface FastEthernet0/2   ! PC1
interface FastEthernet0/3   ! PC2
interface FastEthernet0/4   ! uplink to DSU1 Gi0/0
interface Vlan1
 no ip address
 shutdown

## Switch1

hostname Switch1
interface FastEthernet0/1   ! uplink to DSU1 Gi0/1
interface FastEthernet0/2   ! PC5
interface FastEthernet0/3   ! PC4
interface FastEthernet0/4   ! PC3
interface Vlan1
 no ip address
 shutdown

The switches operate as Layer 2 devices in this topology. No inter-VLAN routing is configured, and no switch management IP is required for the forwarding function demonstrated by the lab.

# 10. PC Configuration

| PC | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 | 10.0.0.2 | 255.0.0.0 | 10.0.0.1 |
| PC1 | 10.0.0.3 | 255.0.0.0 | 10.0.0.1 |
| PC2 | 10.0.0.4 | 255.0.0.0 | 10.0.0.1 |
| PC3 | 20.0.0.2 | 255.0.0.0 | 20.0.0.1 |
| PC4 | 20.0.0.3 | 255.0.0.0 | 20.0.0.1 |
| PC5 | 20.0.0.4 | 255.0.0.0 | 20.0.0.1 |

# 11. Packet Flow Example

Consider PC0 (10.0.0.2) sending a packet to PC3 (20.0.0.2). Because 20.0.0.2 is outside PC0's 10.0.0.0/8 local network, PC0 sends the frame to its default gateway, 10.0.0.1.

| Step | Device | Action |
| --- | --- | --- |
| 1 | PC0 | Sends the packet to its default gateway, DSU1 Gi0/0 at 10.0.0.1. |
| 2 | Switch0 | Forwards the Ethernet frame from PC0 to DSU1. |
| 3 | DSU1 | Recognizes 20.0.0.0/8 as directly connected through Gi0/1 and routes the packet. |
| 4 | Switch1 | Forwards the resulting Ethernet frame to PC3. |
| 5 | PC3 | Receives the packet at 20.0.0.2. |

PC0 → Switch0 → DSU1 → Switch1 → PC3

# 12. Routing Table Concept

Because both networks are directly attached to DSU1, the router does not need manually configured static routes. The expected routing logic is:

| Destination | Route Type | Interface |
| --- | --- | --- |
| 10.0.0.0/8 | Connected | Gi0/0 |
| 20.0.0.0/8 | Connected | Gi0/1 |

# 13. Verification and Testing

Useful router commands:

show ip interface brief
show ip route
show running-config
ping <destination-ip>

Example connectivity tests:

From PC0:
ping 10.0.0.1
ping 20.0.0.1
ping 20.0.0.2

From PC3:
ping 20.0.0.1
ping 10.0.0.1
ping 10.0.0.2

Successful replies verify local gateway connectivity and end-to-end communication between the two LANs.

# 14. Layer 2 vs Layer 3 Roles

| Component | Layer | Primary Function |
| --- | --- | --- |
| Switch0 / Switch1 | Layer 2 | Forward Ethernet frames within each LAN |
| DSU1 | Layer 3 | Route IP packets between 10.0.0.0/8 and 20.0.0.0/8 |
| PCs | End hosts | Generate and receive IP traffic |

# 15. Key Networking Concepts

IPv4 addressing with /8 subnet masks

Default gateways

Directly connected routes

Layer 2 Ethernet switching

Layer 3 IP routing

Inter-LAN communication

Routing table interpretation

Connectivity testing with ICMP ping

# 16. Conclusion

This lab demonstrates how two separate LANs can communicate through a single router. Switch0 and Switch1 provide Layer 2 connectivity for their respective hosts, while DSU1 provides Layer 3 routing between the 10.0.0.0/8 and 20.0.0.0/8 networks. Because both networks are directly connected to the router, no static route or dynamic routing protocol is necessary. Correct default gateway configuration on the PCs is sufficient to enable inter-LAN communication.

# 17. Configuration Summary

| Device | Addressing | Routing Role |
| --- | --- | --- |
| DSU1 | Gi0/0 = 10.0.0.1/8; Gi0/1 = 20.0.0.1/8 | Routes between both directly connected LANs |
| PC0–PC2 | 10.0.0.2–10.0.0.4/8 | Gateway 10.0.0.1 |
| PC3–PC5 | 20.0.0.2–20.0.0.4/8 | Gateway 20.0.0.1 |
| Switch0 | Layer 2 | Connects LAN1 hosts to DSU1 |
| Switch1 | Layer 2 | Connects LAN2 hosts to DSU1 |